README FILE

VERSION 1.05 UPDATE  (10-25-95)

Support for SRAM TESTER.

Added 40-BIT PORT setup of x16 (for 1Mx16 chips)  and x39

for IBM x39 ECC SIMMs.

VERSION 1.04 UPDATE  (6-26-95):

Support for new 3.3V tests.

VERSION 1.03 UPDATE  (1-09-95):

Some improvements in the integrity of the internal data

structure.  No opeartional changes in this version.

VERSION 1.02 UPDATES (10-4-94):

* The speed and size setup for the 40-BIT PORT are now

supported on  the PC COMMUNICATION PACKAGE.  The speed

can be setup in 5nS increments  and the size can be setup

from 256k to 32M (with x18, x32, x33, x36, and x40 bit

configurations as in the normal test mode).

* The PRD setup is now supported on the PC COMMUNICATION

PACKAGE.  This feature works in the same manner as the

PS2 Table [6], in cases when a specific PRD is necessary

to pass a module.

QUICK PC COMMUNICATION SETUP

For advanced SIMCHECK users who hate to read manuals, the following

information has been made available for quick set-up.

* Install the enclosed EPROM in your SIMCHECK.

* While SIMCHECK is OFF, remove the black "dummy" connector

and connect it to the exposed expansion slot of the Serial

Interface Module.  Connect the Serial Interface Module to the

SIMCHECK expansion slot.

* Turn SIMCHECK ON to make sure the green LED gives a steady

glow while the red LED gives steady pulses.  Turn SIMCHECK

OFF.

* Connect the enclosed Serial Cable between the Serial Interface Module

and an available Serial Port (COM1 or COM2) on your PC.

* Create a directory "SIMCHECK" in your hard drive.  Copy

the contents of the enclosed diskette to this directory.

Activate the program by typing "simcheck" and turning

SIMCHECK ON.

You may feel free to experiment with the new set-up fea-

tures available with this package, as well as the familiar

functions that you are accustomed to with SIMCHECK.  Should

you have any questions concerning the operation of the PC

COMMUNICATION PACKAGE, you may always consult your owner's

manual for a detailed description of each function.

As always, we recommend that you read Section 7.14 of your SIMCHECK

Owner's manual.  If you purchased the PC COMMUNICATION PACKAGE separately,

a manual addendum is enclosed which provides more specific information on

its operating procedures.  Please insert the addendum in your SIMCHECK

Owner's Manual at Section 7.14.